Gas Sensor

int redLed = 12;

int greenLed = 11;

int buzzer = 10;

int smokeA0 = A5;

// Your threshold value

int sensorThres = 400;

void setup() {

pinMode(redLed, OUTPUT);

pinMode(greenLed, OUTPUT);

pinMode(buzzer, OUTPUT);

pinMode(smokeA0, INPUT);

Serial.begin(9600);

}

void loop() {

int analogSensor = analogRead(smokeA0);

Serial.print("Pin A0: ");

Serial.println(analogSensor);

// Checks if it has reached the threshold value

if (analogSensor > sensorThres)

{

digitalWrite(redLed, HIGH);

digitalWrite(greenLed, LOW);

tone(buzzer, 1000, 200);

}

else

{

digitalWrite(redLed, LOW);

digitalWrite(greenLed, HIGH);

noTone(buzzer);

}

delay(100);

}

Pulse Sensor

int sensor_pin = 0;

int led_pin = 13;

volatile int heart_rate;

volatile int analog_data;

volatile int time_between_beats = 600;

volatile boolean pulse_signal = false;

volatile int beat[10];         //heartbeat values will be sotred in this array

volatile int peak_value = 512;

volatile int trough_value = 512;

volatile int thresh = 525;

volatile int amplitude = 100;

volatile boolean first_heartpulse = true;

volatile boolean second_heartpulse = false;

volatile unsigned long samplecounter = 0;   //This counter will tell us the pulse timing

volatile unsigned long lastBeatTime = 0;

void setup()

{

pinMode(led_pin,OUTPUT);

Serial.begin(115200);

interruptSetup();

}

void loop()

{

Serial.print("BPM: ");

Serial.println(heart_rate);

delay(200); //  take a break

}

void interruptSetup()

{

TCCR2A = 0x02;  // This will disable the PWM on pin 3 and 11

OCR2A = 0X7C;   // This will set the top of count to 124 for the 500Hz sample rate

TCCR2B = 0x06;  // DON'T FORCE COMPARE, 256 PRESCALER

TIMSK2 = 0x02;  // This will enable interrupt on match between OCR2A and Timer

sei();          // This will make sure that the global interrupts are enable

}

ISR(TIMER2_COMPA_vect)

{

cli();

analog_data = analogRead(sensor_pin);

samplecounter += 2;

int N = samplecounter - lastBeatTime;

if(analog_data < thresh && N > (time_between_beats/5)*3)

{

if (analog_data < trough_value)

{

trough_value = analog_data;

}

}

if(analog_data > thresh && analog_data > peak_value)

{

peak_value = analog_data;

}

if (N > 250)

{

if ( (analog_data > thresh) && (pulse_signal == false) && (N > (time_between_beats/5)*3) )

{

pulse_signal = true;

digitalWrite(led_pin,HIGH);

time_between_beats = samplecounter - lastBeatTime;

lastBeatTime = samplecounter;

if(second_heartpulse)

{

second_heartpulse = false;

for(int i=0; i<=9; i++)

{

beat[i] = time_between_beats; //Filling the array with the heart beat values

}

}

if(first_heartpulse)

{

first_heartpulse = false;

second_heartpulse = true;

sei();

return;

}

word runningTotal = 0;

for(int i=0; i<=8; i++)

{

beat[i] = beat[i+1];

runningTotal += beat[i];

}

beat[9] = time_between_beats;

runningTotal += beat[9];

runningTotal /= 10;

heart_rate = 60000/runningTotal;

}

}

if (analog_data < thresh && pulse_signal == true)

{

digitalWrite(led_pin,LOW);

pulse_signal = false;

amplitude = peak_value - trough_value;

thresh = amplitude/2 + trough_value;

peak_value = thresh;

trough_value = thresh;

}

if (N > 2500)

{

thresh = 512;

peak_value = 512;

trough_value = 512;

lastBeatTime = samplecounter;

first_heartpulse = true;

second_heartpulse = false;

}

sei();

}

Led using array

int timer = 100;           // The higher the number, the slower the timing.
int ledPins[] = {
  2, 7, 4, 6, 5, 3
};       // an array of pin numbers to which LEDs are attached
int pinCount = 6;           // the number of pins (i.e. the length of the array)

void setup() {
  // the array elements are numbered from 0 to (pinCount - 1).
  // use a for loop to initialize each pin as an output:
  for (int thisPin = 0; thisPin < pinCount; thisPin++) {
    pinMode(ledPins[thisPin], OUTPUT);
  }
}

void loop() {
  // loop from the lowest pin to the highest:
  for (int thisPin = 0; thisPin < pinCount; thisPin++) {
    // turn the pin on:
    digitalWrite(ledPins[thisPin], HIGH);
    delay(timer);
    // turn the pin off:
    digitalWrite(ledPins[thisPin], LOW);

  }

  // loop from the highest pin to the lowest:
  for (int thisPin = pinCount - 1; thisPin >= 0; thisPin--) {
    // turn the pin on:
    digitalWrite(ledPins[thisPin], HIGH);
    delay(timer);
    // turn the pin off:
    digitalWrite(ledPins[thisPin], LOW);
  }
}

Touch Sensor

int in = 2;

int out = 13;

int state = HIGH;

int r;

int p = LOW;

long time = 0;

long debounce = 200;

void setup()

{

pinMode(in, INPUT);

pinMode(out, OUTPUT);

}

void loop()

{

r = digitalRead(in);

if (r == HIGH && p == LOW && millis() - time > debounce) {

if (state == HIGH)

state = LOW;

else

state = HIGH;

time = millis();

}

digitalWrite(out, state);

p = r;

}

PIR sensor

void setup() {
  pinMode(2, INPUT); //Pin 2 as INPUT
  pinMode(3, OUTPUT); //PIN 3 as OUTPUT
}

void loop() {
  if (digitalRead(2) == HIGH)
  {
  digitalWrite(3, HIGH);   // turn the LED/Buzz ON
  delay(100);                       // wait for 100 msecond 
  digitalWrite(3, LOW);   // turn the LED/Buzz OFF
  delay(100);                       // wait for 100 msecond 
  }
}

Temperature sensor LM35

float temp;

int tempPin = 0;

void setup() {

Serial.begin(9600);

}

void loop() {

temp = analogRead(tempPin);

// read analog volt from sensor and save to variable temp

temp = temp * 0.48828125;

// convert the analog volt to its temperature equivalent

Serial.print("TEMPERATURE = ");

Serial.print(temp); // display temperature value

Serial.print("*C");

Serial.println();

delay(1000); // update sensor reading each one second

}